The University of Azad Jammu and Kashmir, Muzaffarabad

Name: Alishba Zakir

Roll No: 2024-SE-02

Course: Computer Networks

Course Code: SE-2203

Lab: Router -CLI (2)

Submitted To: Engr. Dr. Asma Javaid

Submitted By: Alishba Zakir

Date: 24 April, 2026

Department Of Software Engineering

Router -CLI

Objective :

The primary goal of this lab is how to connect to networking devices (like routers and switches) and configure them using the Command Line Interface (CLI). Specifically, s will learn to:

Connect hardware using console ports and cables.

Navigate different Cisco IOS modes (User, Privileged, and Global Configuration).

Apply basic security settings and manage configuration files.

Enable :

Used to move from User EXEC mode to Privileged EXEC mode, which allows for advanced access and monitoring.

Configure Terminal :

Used to enter Global Configuration mode from Privileged EXEC mode to change device settings.

Enable password cisco  :

Sets a password to restrict access to the Privileged EXEC mode.

Service  password-encryption :

Encrypts all plain-text passwords (like the enable password) within the configuration file.

Enable secret class  :

Sets a secure, encrypted password for entering Privileged EXEC mode, providing better security than the standard enable password.

End:

Immediately return to Privileged EXEC mode (the Router# prompt) regardless of which configuration sub-mode you are currently in.

Do show running -config   :

Displays the active configuration currently stored in the device's RAM.

Do show startup-config :

Displays the saved configuration stored in the NVRAM, which is used when the device boots up.

Copy running-config startup-config  :

Saves the current active configuration to NVRAM so that changes are not lost after a reboot.